IEEE 754 Fused Dot Product Unit

Complete Bug Fixes & Code Corrections Report

Vinay Kusumanchi (IMT2023608)
Alapati Rajdeep (IMT2023592)

# Overview of Changes

The original code, distributed as separate .v files, contained some bugs that prevented correct simulation and synthesis. All modules have been consolidated into a single Verilog file. The corrected code addresses 10 issues (9 bugs + 1 critical structural defect) and adds IEEE 754 special-case handling, correct rounding increment placement, and an improved significand comparison path.

## Fix 1 - complement_2s.v: Stale State in Loop Variables

| FIX 1 - complement_2s.v - Stale 'pos' and 'condition' variables |  |
| --- | --- |
| File | complement_2s.v - Stale 'pos' and 'condition' variables |
| What was wrong | The registers 'pos' and 'condition' were declared at the top of the module with initial values of 0, but they were never reset inside the always block. In simulation, Verilog always blocks are evaluated multiple times (whenever the sensitivity list triggers). After the first call, 'pos' kept its old non-zero value, making 'condition' turn 1 immediately in every subsequent call. This corrupted every 2's complement conversion after the very first one. |
| Original code | reg [2:0] pos = 0; reg condition = 0; always@(in) begin for(i=0; ...) |
| Fixed code | always@(in) begin pos = 0; condition = 0; // reset at top of block for(i=0; ...) |
| Why it matters | Every module that calls complement_2s (expCompare, expAdjust, KSA_8) was getting wrong negations, causing the entire exponent calculation to produce garbage values. |

## Fix 2 - leadOne.v: 'flag' Never Reset Between Calls

| FIX 2 - leadOne.v - Integer 'flag' not reset |  |
| --- | --- |
| File | leadOne.v - Integer 'flag' not reset |
| What was wrong | The integer 'flag' was initialised to 0 at declaration time but never reset inside the always block. On the very first invocation it worked correctly (flag starts 0, finds the MSB, sets flag=1, stops). On every subsequent call, flag was already 1 so the condition 'if(flag==1) pos=i' was never entered, the module returned the MSB position from the FIRST call forever, regardless of the actual input. |
| Original code | integer i, flag=0; always@(in) begin for(i=47; ...) if(in[i]==1) begin flag=flag+1; if(flag==1) pos=i; end end |
| Fixed code | always@(in) begin flag = 0; pos = 8'd0; // reset here for(i=47; ...) if(in[i]==1 && flag==0) begin pos=i; flag=1; end end |
| Why it matters | The leadOne module drives the normalisation shift amount. A wrong shift amount means the output mantissa is completely wrong for all but the first computation. |

## Fix 3 - pathSelect.v: Wrong Port Width for op_sel (and Wrong Path Logic)

| FIX 3 - pathSelect.v - op_sel declared as [7:0] instead of 1-bit; close/far logic inverted |  |
| --- | --- |
| File | pathSelect.v - op_sel declared as [7:0] instead of 1-bit; close/far logic inverted |
| What was wrong | Two bugs: (1) The input port 'op_sel' was declared as 'input [7:0] aExp,bExp,cExp,dExp,op_sel', effectively 8 bits. Everywhere else in the design op_sel is a single 1-bit wire. This width mismatch causes elaboration errors in synthesis tools and silent mismatches in simulation. (2) The path selection condition 'if(diff<=8'd2 ¦¦ op_sel==0) path_sel=1' is wrong. op_sel==0 means ADDITION, which always goes to the far path, not the close path. |
| Original code | module pathSelect(aExp,bExp,cExp,dExp,path_sel,op_sel); input [7:0]aExp,bExp,cExp,dExp,op_sel; ... if(diff1<=8'd2 ¦¦ op_sel==0) path_sel=1; |
| Fixed code | input [7:0] aExp,bExp,cExp,dExp; input op_sel; // 1-bit ... // 9-bit diff to avoid overflow: if(diff9 <= 9'd2 && op_sel == 1'b1) path_sel = 1; else path_sel = 0; |

## Fix 4 - stickyRound.v: 8-bit mux Used for 1-bit Rounding Signals

| FIX 4 - stickyRound.v - Port width mismatch on rounding mux |  |
| --- | --- |
| File | stickyRound.v - Port width mismatch on rounding mux |
| What was wrong | The original version used the 8-bit mux_2_1 module to select between 1-bit rounding signals (rndPos, rndNeg, rndNearEven). An 8-bit mux driving a 1-bit output register causes an elaboration/port-width error in simulation. The mux_2_1 module can only select one of two 8-bit buses; it cannot pick one of three 1-bit values. |
| Original code | // Used 8-bit mux_2_1 to select rounding mode: mux_2_1 m1(..., rndPos, rndNeg, rndMode[0], ...); mux_2_1 m2(..., rndNearEven, ..., rndMode[1], ...); |
| Fixed code | // New 1-bit 4-to-1 mux: assign rndPos = sticky ? signComp : andOp1; assign rndNeg = sticky ? sign : andOp2; assign rndNearEven = sticky ? guard : andOp3; mux_4_1 m4(1'b0,rndPos,rndNeg,rndNearEven, rndMode[1],rndMode[0], rndUp); |
| Why it matters | Without the correct rounding mux, the rounding mode had no effect, the unit always rounded in the same direction regardless of the rnd_mode input. The mux_4_1 module was also added to the design to support this fix. |

## Fix 5 - mainMod.v: Output Never Assembled as IEEE 754

| FIX 5 - mainMod.v - Missing IEEE 754 field packing |  |
| --- | --- |
| File | mainMod.v - Missing IEEE 754 field packing |
| What was wrong | The original mainMod computed partial values (sign, exponent, mantissa) but the final 'out' assignment just copied the wire 'out1' which was itself poorly defined. The three IEEE 754 fields (sign, exponent, mantissa) were never concatenated into the 32-bit output. |
| Original code | // out was never properly assembled: always@(out1) out = out1; // out1 is [31:0] but normalize returns [47:0] |
| Fixed code | // Correct IEEE 754 packing: always @(*) begin ... out = {sign, final_exp, mantissa_out}; end |
| Why it matters | The output was meaningless, it never produced a valid IEEE 754 single-precision number regardless of what the internal computation calculated correctly. |

## Fix 6 - mainMod.v: normalize Module Called with Wrong Output Port Width

| FIX 6 - mainMod.v - Incorrect port for normalize output |  |
| --- | --- |
| File | mainMod.v - Incorrect port for normalize output |
| What was wrong | normalize was called with a [31:0] wire 'out1' as the output, but the normalize module produces a [47:0] result. The lower bits (the mantissa) were being extracted from the wrong bit positions, and the upper bits were simply discarded. |
| Original code | wire [31:0] out1; normalize nn4(final_sum, norm_shift1, out1, msb); // width mismatch |
| Fixed code | // Keep full 48-bit result; extract mantissa from correct bits: wire [47:0] rounded = rnd_up ? (norm_sig + 48'h1000000) : norm_sig; wire [22:0] mantissa_out = rounded[46:24]; |
| Why it matters | Every mantissa value would be wrong, the bits would be 24 positions off, producing a completely incorrect fraction field in the output. |

## Fix 7 - mainMod.v: Exponent from expAdjust Never Used in Output

| FIX 7 - mainMod.v - expAdjust output not wired to final output |  |
| --- | --- |
| File | mainMod.v - expAdjust output not wired to final output |
| What was wrong | expAdjust was instantiated and computed a 'finalExp', but this value was never connected to the output. The output used an intermediate wire 'exponent' set by a broken always block: 'if(A==B && C==D) exponent=exp+1', this makes no mathematical sense for a dot product. |
| Original code | // Broken exponent logic in original: always@(A or B or C or D or exp) begin if(A==B && C==D) exponentTemp=exp+1; else exponentTemp=exp; end |
| Fixed code | // Correct exponent formula (no expAdjust needed): wire [8:0] final_exp_9 = exp_large + {1'b0,msb_pos} - 9'd46; wire [7:0] final_exp = final_exp_9[7:0]; // Used directly in: out = {sign, final_exp, mantissa_out}; |
| Why it matters | The exponent field of the output was always wrong, a properly rounded mantissa with a wrong exponent still produces the completely wrong floating-point value. |

## Fix 8 - KSA.v: Entire Module Is an FPGA Debug Stub

| FIX 8 - KSA.v - 32-bit KSA replaced by Xilinx VIO debug probe (CRITICAL) |  |
| --- | --- |
| File | KSA.v - 32-bit KSA replaced by Xilinx VIO debug probe (CRITICAL) |
| What was wrong | The 32-bit Kogge-Stone Adder module KSA.v had its entire port interface commented out and replaced with a Xilinx Virtual I/O (VIO) debug core ('vio_0'). The module only had 'input clk' and the actual a/b/cin/sum/cy ports were commented away. This module is completely non-functional for simulation or synthesis, it is an FPGA hardware debugging stub left behind. |
| Original code | module KSA(clk); // ← ALL REAL PORTS COMMENTED OUT input clk; vio_0 your_instance_name ( .clk(clk), .probe_in0(sum), ... // FPGA-only debug IP ); |
| Fixed code | // The 32-bit KSA is not needed (all exp arithmetic is 8-bit). // KSA.v is EXCLUDED from the design. // KSA_8.v (the 8-bit version, which IS correct) // is used for all exponent operations. |

## Fix 9 - expCompare.v: Exponent Sum Overflow and Wrong Bias Subtraction

| FIX 9 - expCompare.v - 8-bit overflow on exponent sum; bias subtracted wrong number of times |  |
| --- | --- |
| File | expCompare.v - 8-bit overflow on exponent sum; bias subtracted wrong number of times |
| What was wrong | Two bugs: (1) The original code added expA+expB using 8-bit KSA_8. Since expA and expB can each be up to 254, their sum can reach 508 which does not fit in 8 bits, the top bit is silently dropped, causing an overflow. (2) The bias (127) was subtracted only once from the combined sum of both products instead of once per product, giving an exponent that was 127 too large. |
| Original code | // 8-bit — overflows at sum > 255: KSA_8 k1(aExp,bExp,1'b0,abExp,cy_ab); // Bias subtracted only once at end: KSA_8 k4(muxOut,bias,1'b0,exp,cout); |
| Fixed code | // 9-bit, no overflow: wire [8:0] abSum9 = {1'b0,aExp} + {1'b0,bExp}; // Subtract bias from EACH product separately: wire [8:0] abExp_real = abSum9 - 9'd127; wire [8:0] cdExp_real = cdSum9 - 9'd127; |
| Why it matters | The exponent comparison and alignment shift were based on a wrong value. Nearly all non-trivial inputs would produce the wrong exponent and therefore the wrong result. This is one of the most impactful bugs in the original code. |

## Fix 10 - mainMod.v: Rounding Increment Applied at Wrong Bit Position

| FIX 10 - mainMod.v - Rounding increment added at bit 0 instead of bit 24 |  |
| --- | --- |
| File | mainMod.v - Rounding increment added at bit 0 instead of bit 24 |
| What was wrong | The original code (where rounding was attempted at all) added 48'd1 to the normalised significand. This adds 1 at bit 0 of the 48-bit value, which is far below the 23-bit mantissa field stored at bits [46:24]. Adding 1 at bit 0 has no effect whatsoever on the mantissa, the rounding was completely non-functional. |
| Original code | // WRONG: adds at bit 0 — below mantissa, has no effect: rounded = norm_sig + 48'd1; |
| Fixed code | // CORRECT: adds 1 ULP at bit 24 — inside mantissa: rounded = rnd_up ? (norm_sig + 48'h1000000) : norm_sig; // mantissa_out = rounded[46:24]; |
| Why it matters | The rounding operation had zero effect on the output. All results were silently unrounded, violating IEEE 754 round-to-nearest semantics for all four rounding modes. |

# Additional Improvements Added

## A. IEEE 754 Special-Case Handling

The original code had no special-case handling. The corrected code adds proper IEEE 754 special cases directly in mainMod using combinational wire logic:(This is based from the riscv-spec-v2.1 document we were shared)

- NaN propagation: if any input is NaN (exponent all-1s, non-zero fraction), output is Quiet NaN (0x7FC00000).

- Infinity × Zero: returns Quiet NaN (invalid operation, per IEEE 754 §7.2).

- Infinity propagation: if any non-zero input product involves infinity, output is signed infinity.

- Zero result: if both products are zero or the sum cancels to zero, output is +0.0.

## B. Correct Significand Magnitude Comparison

The original code used the CSA carry-out 'signficantCompare' module to determine which product is larger, comparing two single carry-out bits. This gives an unreliable result, two CSA carry-outs tell you almost nothing about relative magnitude. The corrected code uses the correct approach: compare the real product exponents first, and only compare the 48-bit integer significands when the exponents are equal:

wire exp_eq   = (eAB == eCD); wire exp_gt   = (eAB >  eCD); wire sig_ge   = (multAB >= multCD);  // only needed when exp equal wire exp_comp = exp_gt | (exp_eq & sig_ge);

## C. Direct Exponent Formula: Eliminates the Broken expAdjust Path

Instead of relying on the broken expAdjust chain (which was itself never wired to the output), the corrected mainMod computes the final exponent directly using the standard formula for a fused multiply-add:

wire [8:0] final_exp_9 = exp_large + {1'b0,msb_pos} - 9'd46; wire [7:0] final_exp   = final_exp_9[7:0];

The constant 46 is the nominal MSB position for the product of two 24-bit IEEE 754 significands (implicit leading 1 at bit 23, so product MSB is at bit 23+23=46). The msb_pos from leadOne adjusts for the actual normalisation needed after addition.

## D. Single Consolidated File

The professor provided separate .v files for each module. All modules are combined into a single well-commented Verilog file with the complete bug-fix list at the top, making it easy to compile, simulate, and review in any simulator (Vivado, ModelSim, Icarus Verilog, etc.).

# Modules Verified Correct (No Changes)

The following modules from the original code were verified to be functionally correct and are integrated into the combined file without modification:

- FA / HA - Standard full adder and half adder gate-level implementations. Correct.

- dadda - 24×24 Dadda multiplier reduction tree (stages 1–8). Functionally correct; only reformatted for readability.

- csa4_2 - 4:2 Carry-Save Adder (48-bit). Correct.

- grCarryGen / grCarryPropagate - Kogge-Stone helper cells. Correct.

- KSA_8 - 8-bit Kogge-Stone Adder. Correct (note: 32-bit KSA.v was broken; see Fix 8).

- inverter - Conditional bitwise inverter. Port order (control, in, out) verified against all call sites.

- mux_2_1_47bits - 48-bit 2-to-1 mux. Correct.

- mux_normalize - 48-bit 2-to-1 mux for normalise path. Correct.

- signLogic - Result sign determination logic. Correct.

- signficantCompare - Carry-out comparator. Correct in isolation (improved usage in mainMod, see Fix B).

- opSelect - Determines effective add/subtract. Correct.

- shifter_48b - Right-shift aligner with guard/round/sticky. Correct.

- normalize - Left-shift normaliser. Correct (output width fix applied in mainMod, not here).

- expInvert - Conditional exponent negation. Correct.